Upgrading Firmware on the WMLS11B on a Computer with Vista

by ilovepool28@yahoo.com

There is a way! This worked for me:

- Download the most recent firmware (currently 2.3.6) from the "Files" section of this group.    Save it in a folder you can easily locate later.

- Run the Setup Wizard from the disk that came with the WMLS11B. It will give you a message that you need to run the program as an administrator, but that's okay. By this time, it has done what you needed it to do and you can close out of the program.

- Connect your WMLS11B to your computer via Ethernet.

- Click on the Windows icon, then CONTROL PANEL, then PROGRAMS. Under "Programs and Features," click on USE AN OLDER PROGRAM WITH THIS VERSION OF WINDOWS. This will start the Program Compatibility Wizard.

- Click NEXT, then click I WANT TO CHOOSE FROM A LIST OF PROGRAMS. Click NEXT and wait while it searches your programs. Eventually, it will ask which program you want to run with compatibility settings. Click on LINKSYS WIRELESS-B MEDIA LINK, then click NEXT.

- From the list, choose WINDOWS 2000, then click NEXT.

- Don't choose anything from the list that comes up now. Just click NEXT.

- Check RUN AS ADMINISTRATOR and click NEXT.

- Click NEXT again. It will test the Setup Wizard, which should now be functional and at the Firmware Upgrade screen.

- Your WMLS11B should be listed under "Device Name," along with its MAC address. If it isn't, check your Ethernet connection, your firewall settings, and posts to this group for troubleshooting.

- If your system has been located and listed by the Setup Wizard, click BROWSE and locate the firmware file you downloaded in step 1. If you chose 2.3.6, the file would be called wmls11b_fw_v2. 3.6_US_code. bcd. When the firmware file shows up in your Firmware box, click UPGRADE.

- You may receive a warning that your WMLS11B already has a newer firmware version installed.    Ignore that and continue to upgrade.

- Go back to the Program Compatibility Wizard window and click on YES. This will allow you to skip all of the compatibility steps should there ever be another upgrade.

That should do it! You can still use Twonky as your Media Server and you will now be able to get the Linksys Super Service and do anything else those XP guys can do!